| Form 3160-5 UNITED STATES (June 2019) DEPARTMENT OF THE INTERIOR BUREAU OF LAND MANAGEMENT SUNDRY NOTICES AND REPORTS ON WELLS Do not use this form for proposals to drill or to re-enter an abandoned well. Use Form 3160-3 (APD) for such proposals. | FORM APPROVED OMB No. 1004-0137 Expires: December 31, 2024 |
| --- | --- |
|  | 5. Lease Serial No. NM-100558 |
|  | 6. If Indian, Allottee or Tribe Name |

| SUBMIT IN TRIPLICATE - Other instructions on page 2 |  | 7. If Unit of CA/Agreement, Name and/or No. |
| --- | --- | --- |
| 1. Type of Well ✔ Oil Well Gas Well Other |  |  |
|  |  | 8. Well Name and No. RDX Federal 15-1 |
| 2. Name of Operator WPX Energy Permian, LLC |  | 9. API Well No. 30-015-35955 |
| 3a. Address 333 West Sheridan Ave. Oklahoma City, Ok 73102 | 3b. Phone No. (include area code) (405) 228-2480 | 10. Field and Pool or Exploratory Area Brushy Draw, Delaware East |
| 4. Location of Well (Footage, Sec., T.,R.,M., or Survey Description) D, Sec. 15, T26S, R30E 330 FNL 330 FWL |  | 11. Country or Parish, State Eddy County, NM |

- CHECK THE APPROPRIATE BOX(ES) TO INDICATE NATURE OF NOTICE, REPORT OR OTHER DATA

| TYPE OF SUBMISSION | TYPE OF ACTION |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Notice of Intent | Acidize Alter Casing |  | Deepen Hydraulic Fracturing | Production (Start/Resume) Reclamation | Water Shut-Off Well Integrity |
| ✔ Subsequent Report | Casing Repair Change Plans | ✔ | New Construction Plug and Abandon | Recomplete Temporarily Abandon | Other |
| Final Abandonment Notice | Convert to Injection |  | Plug Back | Water Disposal |  |

- Describe Proposed or Completed Operation: Clearly state all pertinent details, including estimated starting date of any proposed work and approximate duration thereof. If the proposal is to deepen directionally or recomplete horizontally, give subsurface locations and measured and true vertical depths of all pertinent markers and zones. Attach the Bond under which the work will be performed or provide the Bond No. on file with BLM/BIA. Required subsequent reports must be filed within 30 days following completion of the involved operations. If the operation results in a multiple completion or recompletion in a new interval, a Form 3160-4 must be filed once testing has been completed. Final Abandonment Notices must be filed only after all requirements, including reclamation, have been completed and the operator has determined that the site is ready for final inspection.)

JMR Services, LLC plugged this well on February 7, 2026

Please see attached the P&A Subsequent Report.

| 14. I hereby certify that the foregoing is true and correct. Name (Printed/Typed) | Title |
| --- | --- |
| Signature | Date |

THE SPACE FOR FEDERAL OR STATE OFICE USE

| Approved by | Title | Date |
| --- | --- | --- |
| Conditions of approval, if any, are attached. Approval of this notice does not warrant or certify that the applicant holds legal or equitable title to those rights in the subject lease which would entitle the applicant to conduct operations thereon. | Office |  |

Title 18 U.S.C Section 1001 and Title 43 U.S.C Section 1212, make it a crime for any person knowingly and willfully to make to any department or agency of the United States

any false, fictitious or fraudulent statements or representations as to any matter within its jurisdiction.

(Instructions on page 2)

# P&A Subsequent Report

Devon Energy Production Company, L.P.

RDX Federal 15-1

30-015-35955

1/09/26: MIRU. Check well pressures: Tubing 0 psi, Casing 80 psi, Surface 0 psi. RIH and tagged existing CIBP @6383’.  Unable to establish circulation with 205 bbl.  Will test CIBP. SDFD

1/10/26:  Check well pressures: 0 psi. Set packer @ 6380 and P/T CIBP; ok.  With packer tested below 6042; well went on a vacuum after 40 bbls. SDFD.

1/12/26: Check well pressures: 0 psi.  Work to isolate casing leaks.  Determine bad casing from 6239’ to 5714’.  Wait on BLM orders.

1/13/26: Check well pressures: 0 psi.  RU JSI wireline.

Plug #1:Dump bail 40’ of cement (2 runs) above existing CIBP @ 6383’.  RD JSI.  SDFD.

1/14/26: Check well pressures: 0 psi.  RIH and tag cement @ 6345’.

Plug #2:  Squeeze 150 sxs class C cement from 6345’ to 5060’.  WOC.  Tag Plug #2 @ 5755’. SDFD.

1/15/26: Check well pressure: 0 psi.  Pressure test casing to 700# for 30 min; good.  David Alvarado, NMOCD, approved to spot 75 sxs class C cement to bring from 5755’ to 5060’ and then continue plugging operation.  Bubble tested casing to make sure plug was good.

Plug #3:  spot 75 sxs class C cement from 5755’ to 5005’.  WOC.  Tag TOC @ 4930’.  SDFD.

1/16/26: Check well pressure: 0 psi.  RU 3rd party wireline.  Run CBL; found TOC @ 3190’.

Plug #4: Spot 25 sxs class C cement from 4566’ to 4313’.

Plug #5: Spot 25 sxs class C cement from 3619’ to 3366’.   SDFD.

1/17/26: Check well pressure: 0 psi.  Tagged Plug #5 @ 3360’.  RIH with 5.5” packer to 1239’.  Perforate @ 1594’.

Plug #6:  Squeeze 40 sxs class C cement from 1594’ to 1480’.  SDFD.

1/19/26:  Check well pressure: 0 psi.  Tagged Plug #6 @ 1410’.  Perforate @ 1100.

Plug #7: Squeeze 40 sxs class C cement from 1100’ to 700’.  WOC.  Tag TOC @ 690’.  Perforate @ 100’.

Plug #8: Squeeze 40 sxs class C cement from 100’ to surface. SDFD.

1/20/26: Check well pressure: 0 psi.  David Mervine, BLM, called and stated that cement on Plug #7 should have been more cement but was approved with 40 sxs which is what was pumped.  JMR has to drill out to 1100’ due to BLM error.

1/21/26 through 1/30/26.  Drilled down to 1050’.

1/31/26:  Check well pressure: 0 psi.  Continued to drill out cement; broke through past 1100’.  Circulate well clean.  Perforate @ 1090’.

Plug #9:  Squeeze 120 sxs class C cement from 1090’ to 700’. SDFD.

2/1/26:  Check well pressure: 0 psi.  Tagged TOC @ 680’.

Plug #10: Circulated 30 sxs class C cement from 160’ to surface.  SDFD.

2/2/26: Check well pressure: 0 psi.  Verified cement to surface.

2/7/26: Dig out cellar.  Cut off wellhead. Set above ground P&A marker.